COMBIOS was originally furnished by AA4RE, and modified extensively

by WA7MXZ, WA7MBL, and W0RLI.

These serial device drivers replace the standard Int 14 drivers

included in the IBM ROM BIOS.

COMBIOS  supports the standard COM1 & COM2 ports.

COM1BIOS supports only COM1.

COM2BIOS supports only COM2.

QUADBIOS supports COM3 - COM7 on a Quadram Quadport AT board.

COMXBIOS supports COM25 and COM26 for "serial link" - W0RLI

GFIBIOS  supports COM1 - COM4 for the GFI 4 port board,

with aa4re hardware mod.

Configured for COM1 - COM4 = IRQ3

These drivers may be loaded in any order and combination for

whatever serial port boards you have. Take care that only one

board has each COM port - that is, no duplicate COM1 for example.

When using the Quadram board, you may only have one standard serial

port in use, thus QUADBIOS may be loaded in conjunction with either

COM1BIOS or COM2BIOS for a total of 6 ports.

Setup info: (example for W0RLI AT clone)

Load the drivers and set port speed, etc. in your autoexec.bat file:

COM1BIOS

QUADBIOS

MBMODE COM1:12,n,8,1

MBMODE COM3:48,n,8,1

MBMODE COM4:48,n,8,1

MBMODE COM5:48,n,8,1

MBMODE COM6:48,n,8,1

MBMODE COM7:96,e,7,1

Programming information:

The routines are called in the same manner as the standard Int 14 calls

are made.  However, there are more possible calls as determined by the

value passed in AH.

The port number should be passed in DX.  (0-COM1 1-COM2, etc)

Valid values for DX:

COMBIOS  (0,1)

COM1BIOS (0)

COM2BIOS (1)

QUADBIOS (2,3,4,5,6)

GFIBIOS  (0,1,2,3)

COMXBIOS (24,25)

AH = 0	   Initialize

---------------------

This initializes the communications port.  AL contains baud rate, parity,

stop bit, and word length as in the standard BIOS call.  On return, AH

contains the Line Status Register information (except bit 0 - Data ready

tells whether there is information in the extended buffer rather than the

SIO chip receive buffer).  AL contains the Modem Status Register.

AH = 1	   Send Character

-------------------------

AL contains the character to send.  Return values are the same as above.

AH = 2	   Receive Character

----------------------------

On return, AL contains the character received.	AH contains the contents

of the LSR at the time the last character was received from the TNC (not

necessarily the character this routine returns).  Again, AH bit 0 indicates

if there is additional characters in the buffer.  This routine does NOT

timeout while waiting for a character.

AH = 3	   Return Status

------------------------

Return values are the same as returned by the Initialize routine.

AH = 4	   Inquiry

------------------

AH returns 0AAH.  AL returns 055H.  This call is used to see if the

driver has been loaded for a particular port.

Port setup for GFI 4 port board using GFIBIOS:

(Add the diodes and resister per aa4re instructions)

PORT	  ADDRESS	 INTERRUPT

-----	  ----------	 ---------

COM 1	  3F8-3FF	 IRQ3

COM 2	  2F8-2FF	 IRQ3

COM 3	  3E8-3EF	 IRQ3

COM 4	  3E0-3E7	 IRQ3