This file is an attempt  at explaining the internals of the FreeType

rasterizer.   This  component is  quite  general  purpose and  could

easily  be  integrated into  other  programs  (but  still under  the

current license).

--------------------------------------------------------------------

HOWs and WHYs of the FreeType rasterizer

by David Turner

I. Introduction

II. Rendering Technology

III. Implementation Details

IV. Gray-Level Support

I. Introduction:

A  rasterizer is  a library  in charge  of converting  a vectorial

representation of a shape  into a bitmap.  The FreeType rasterizer

has been developed  to render the glyphs found  in TrueType files,

made up of segments and second-order Beziers.  This document is an

explanation of its design and implementation.

Though these  explanations start from  the basics, a  knowledge of

common rasterization techniques is assumed.

II. Rendering Technology:

1. Requirements:

We  will assume  that  all scaling/rotating/hinting/whatever  have

been  already  done.  The  glyph  is  thus  described, as  in  the

TrueType specification, by a list  of points.  Each point has an x

and y  coordinate, as  well as a  flag that indicates  whether the

point is _on_ or _off_ the curve.

More precisely:

- All point  coordinates are  in the 26.6  fixed float  format as

defined by the specification.  The orientation used is:

^ y

|         reference orientation

|

*----> x

0

This means that the  'distance' between two neighbouring pixels

is 64 pixel 'units' (1 unit = 1/64th of a pixel).

Note  that, for the  rasterizer, pixel  centers are  located at

integer coordinates, i.e.  (0.0,  0.0) is the coordinate of the

origin's  center  (unlike  what  happens  within  the  TrueType

bytecode interpreter).

A pixel line in the target bitmap is called a 'scanline'.

- A  glyph  is usually  made  of  several  contours, also  called

outlines.  A contour is simply  a closed curve that delimits an

outer  or inner  region of  the glyph.   It is  described  by a

series of successive points of the points table.

Each point of  the glyph has an associated  flag that indicates

whether it  is "on"  or "off" the  curve.  Two  successive "on"

points indicate a line segment joining the two points.

One  "off" point  amidst  two "on"  points  indicates a  second

degree  Bezier parametric  arc, defined  by these  three points

(the "off" point being the control point, and the "on" ones the

start and end points).

Finally, two  successive "off" points forces  the rasterizer to

create, during  rendering, an "on" point amidst  them, at their

exact  middle.   This  greatly  facilitates the  definition  of

successive Bezier arcs.

*            # on

* off

__---__

#-__                      _--       -_

--__                _-            -

--__           #               \

--__                        #

-#

Two "on" points

Two "on" points       and one "off" point

between them

*

#            __      Two "on" points with two "off"

\          -  -     points between them. The point

\        /    \    marked '0' is the middle of the

-      0      \   "off" points, and is a 'virtual'

-_  _-       #   "on" point where the curve passes.

--             It does not appear in the point

*              list.

The FreeType rasterizer, as intended to render TrueType glyphs,

does not support  third order Beziers, usually found  in Type 1

fonts.  Type 1  support may lead to further  development of the

engine.

The parametric form of a second-order Bezier is:

P(t) = (1-t)^2*P1 + 2*t*(1-t)*P2 + t^2*P3

with t a real number in the range [0..1]

P1 and P3 are the endpoints, P2 the control point.

Note  that  the  rasterizer  does  not use  this  formula.   It

exhibits,  however, one  very useful  property of  Bezier arcs:

each point  of the curve is  a weighted average  of the control

points.

As all  weights are positive and  always sum up  to 1, whatever

the value of t, each arc point lies within the triangle defined

by the arc's three control points.

2. Profiles and Spans:

The  following   is  a  basic   explanation  of  the   _kind_  of

computations  made by  the rasterizer  to build  a bitmap  from a

vector  representation.  Note that  the actual  implementation is

slightly different, due to performance tuning and other factors.

However, the following ideas remain in the same category, and are

more convenient to understand.

a. Sweeping the shape:

The best way  to fill a shape is to decompose  it into a number

of simple horizontal segments, then  turn them on in the target

bitmap.  These segments are called 'spans'.

__---__

_--       -_

_-            -

-               \

/                 \

/                   \

|                     \

__---__         Example: filling a shape

_----------_                with spans.

_--------------

----------------\

/-----------------\    This is typically done from the top

/                   \   to the bottom of the shape, in a

|           |         \  movement called a "sweep".

V

__---__

_----------_

_--------------

----------------\

/-----------------\

/-------------------\

|---------------------\

In  order to  draw  a  span, the  rasterizer  must compute  its

coordinates,   which   are   simply   the   shape's   contours'

x-coordinates taken on the y scanlines.

/---/    |---|   Note that there are usually several

/---/     |---|   spans per scanline.

|        /---/      |---|

|       /---/_______|---|   When rendering this shape to the

V      /----------------|   current scanline y, we must compute

/-----------------|   the x values of the points a, b, c,

a /----|         |---|   and d.

- - - *     * - - - - *   * - - y -

/     / b       c|   |d

/---/    |---|

/---/     |---|  And then turn on the spans a-b and c-d.

/---/      |---|

/---/_______|---|

/----------------|

/-----------------|

a /----|         |---|

- - - ####### - - - - ##### - - y -

/     / b       c|   |d

b. Decomposing outlines into profiles:

For  each scanline  during  the sweep,  we  need the  following

information:

o The number of  spans on the current scanline,  given by the

number of shape points intersecting the scanline (these are

the points a, b, c, and d in the above example).

o The x coordinates of these points.

These  are  computed  before  the  sweep,  in  a  phase  called

'decomposition' which converts the glyph into *profiles*.

Put it simply, a "profile" is a contour's portion that can only

be either ascending or descending, i.e., it is monotonic in the

vertical direction (we will also say Y-monotonic).  There is no

such thing as a horizontal profile, as we shall see.

Here are a few examples:

this square

1         2

---->----     is made of two

|         |                       |         |

|         |       profiles        |         |

^         v                       ^    +    v

|         |                       |         |

|         |                       |         |

----<----

up         down

this triangle

P2                             1          2

|\        is made of two       |         \

^  | \  \                         |          \

| |   \  \      profiles         |            \      |

|  |    \  v                  ^   |             \     |

|      \                    |  |         +     \    v

|       \                   |  |                \

P1 ---___   \                     ---___            \

---_\                          ---_         \

<--__     P3                   up           down

A more general contour can be made of more than

two profiles:

__     ^

/  |   /  ___          /    |

/   |     /   |        /     |       /     |

|    |    /   /    =>  |      v      /     /

|    |   |   |         |      |     ^     |

^  |    |___|   |  |      ^   +  |  +  |  +  v

|  |           |   v      |                 |

|           |          |           up    |

|___________|          |    down         |

<--               up              down

Successive  profiles are always  joined by  horizontal segments

that are not part of the profiles themselves.

Note that  for the rasterizer,  a profile is simply  an _array_

that  associates  one  horizontal  *pixel* coordinate  to  each

bitmap *scanline*  crossed by the  contour's section containing

the profile.  Note also that profiles are *oriented* up or down

along the glyph's original flow orientation.

In other  graphics libraries, profiles are  also called 'edges'

or 'edgelists'.

c. The Render Pool:

FreeType has  been designed  to be able  to run well  on _very_

light systems, including embedded systems with very few memory.

A render pool will be  allocated once; the rasterizer uses this

pool for all its needs  by managing this memory directly in it.

The algorithms  that are used  for profile computation  make it

possible to use the pool  as a simple growing heap.  This means

that this  memory management is actually easy,  and faster than

any kind of malloc()/free() combination.

Moreover,  we'll see later  that the  rasterizer is  able, when

dealing with profiles too large and numerous to lie all at once

in the  render pool,  to immediately decompose  recursively the

rendering process into  independent sub-tasks, each taking less

memory to be performed (see "sub-banding" below).

The render  pool doesn't  need to be  large.  A 4kByte  pool is

enough  for nearly  all renditions,  though nearly  100% slower

than a  more confortable  16 or 32kByte  pool (that  was tested

with complex glyphs at sizes over 500 pixels).

d. Computing Profiles Extents:

Remember that  a profile is an array,  associating a _scanline_

to the x pixel coordinate of its intersection with a contour.

Though it's  not exactly how the FreeType  rasterizer works, it

is convenient to  think that we need a  profile's height before

allocating it in the pool and computing its coordinates.

The profile's height is the  number of scanlines crossed by the

Y-monotonic  section of  a contour.   We thus  need  to compute

these sections from the  vectorial description.  In order to do

that,  we  are  obliged  to  compute  all  (local  and  global)

Y-extrema of the glyph (minima and maxima).

P2             For instance, this triangle has only

two Y-extrema, which are simply

|\

| \               P2.y  as an Y-maximum

|   \              P3.y  as an Y-minimum

|    \

|      \            P1.y is not an Y-extremum (though it is

|       \           a X-minimum, which we don't need).

P1 ---___   \

---_\

P3

Note  that the  extrema are  expressed in  pixel units,  not in

scanlines.   The triangle's  height is  certainly (P3.y-P2.y+1)

pixel  units,  but  its   profiles'  heights  are  computed  in

scanlines.  The exact conversion is simply:

- min scanline = FLOOR  ( min y )

- max scanline = CEILING( max y )

A problem arises  with Bezier Arcs.  While a  segment is always

necessarily Y-monotonic (i.e., flat, ascending, or descending),

which makes extrema computations easy,  the ascent of a arc can

vary between its control points.

P2

*

# on

* off

__-x--_

_--       -_

P1  _-            -       A non Y-monotonic Bezier arc.

#               \

-        The arc goes from P1 to P3.

\

\  P3

#

We first need  to be able to easily  detect non-monotonic arcs,

according to their control  points.  I will state here, without

proof, that the monotony condition can be expressed as:

P1.y <= P2.y <= P3.y   for an ever-ascending arc

P1.y >= P2.y >= P3.y   for an ever_descending arc

with the special case of

P1.y = P2.y = P3.y     where the arc is said to be 'flat'.

As you  can see,  these conditions can  be very  easily tested.

They are,  however, extremely important,  as any arc  that does

not satisfy them necessarily contains an extremum.

Note also  that a  monotonic arc can  contain an  extremum too,

which is then one of its 'on' points:

P1           P2

#---__   *         P1P2P3 is ever-descending, but P1

-_           is an Y-extremum.

-

---_    \

->   \

\  P3

#

Let's go back to our previous example:

P2

*

# on

* off

__-x--_

_--       -_

P1  _-            -       A non Y-monotonic Bezier arc.

#               \

-        here we have

\              P2.y >= P1.y &&

\  P3         P2.y >= P3.y      (!)

#

We need  to compute  the y-maximum  of this arc  to be  able to

compute a profile's  height (the point marked by  an 'x').  The

arc's equation indicates that a direct computation is possible,

but only if  we care about performing at  least one square root

extraction (with sufficient  accuracy).  This computation being

much too  slow for us, we  use an alternative  method that will

become very handy in the following chapters:

Bezier  arcs have  the special  property of  being  very easily

decomposed  into  two  other  sub-arcs,  which  are  themselves

Beziers arcs.  Moreover,  it is easy to prove  that there is at

most  one Y-extremum  on  each Bezier  arc  (for second  degree

ones).

For instance,  the following arc P1P2P3 can  be decomposed into

two sub-arcs Q1Q2Q3 and R1R2R3 that look like:

P2

*

# on  curve

* off curve

Original Bezier Arc P1P2P3

__---__

_--       --_

_-             -_

-                 -

/                   \

/                     \

#                       #

P1                         P3

P2

*

Q3               Decomposed into two subarcs

Q2                R2

*   __-#-__   *           Q1Q2Q3 and R1R2R3 with

_--       --_

_-       R1    -_       Q1 = P1          R3 = P3

-                 -      Q2 = (P1+P2)/2   R2 = (P2+P3)/2

/                   \

/                     \        Q3 = R1 = (Q2+R2)/2

#                       #

Q1                         R3    Note that Q2, R2, and Q3=R1

are on a single line which is

tangent to the curve.

We  have  then  decomposed  a non-Y-monotonic  bezier  into  two

smaller sub-arcs.  Note that in the above drawing, both sub-arcs

are monotonic, and that the extremum is then Q3=R1.  However, in

a  more general  case,  only  one sub-arc  is  guaranteed to  be

monotonic.  Getting back to our former example:

Q2

*

__-x--_ R1

_--       #_

Q1  _-        Q3  -   R2

#               \ *

-

\

\  R3

#

Here, we see that,  though Q1Q2Q3 is still non-monotonic, R1R2R3

is ever  descending: we  thus know that  it doesn't  contain the

extremum.  We  can then  re-subdivide Q1Q2Q3 into  two sub-arcs,

and go on recursively,  stopping when we encounter two monotonic

subarcs, or when the subarcs become simply too small.

We  will  finally find the y-extremum.  Note that the iterative

process of finding an extremum is called 'flattening'.

e. Computing Profiles coordinates:

Once we have the height  of each profile, we're able to allocate

it in  the render pool.  We  now have to  compute its coordinate

for each scanline.

In the case of segments, the computation is straightforward, and

uses good  old Euclide (also  known as Bresenham  ;-).  However,

for Bezier arcs, things get a little more complicated.

We assume  that all Beziers that  are part of a  profile are the

result of  'flattening' the curve, which means  that they're all

y-monotonic (ascending  or descending, and never  flat).  We now

have  to  compute the  arcs'  intersections  with the  profile's

scanlines.  One way is to  use a similar scheme to 'flattening',

called 'stepping'.

Consider this arc, going from P1 to

---------------------      P3.  Suppose that we need to

compute its intersections with the

drawn scanlines.  Again, this is

---------------------      feasible directly, if we dare

to compute one square root per

* P2         _---# P3  scanline (how great!).

------------- _--  --

_-

_/               Rather, it is still possible to use

---------/-----------      the decomposition property in the

/                  same recursive way, i.e. subdivide

|                   the arc into subarcs until these

------|--------------      get too small to cross more than

|                    one scanline!

|

-----|---------------      This is very easily done using a

|                      rasterizer-managed stack of

|                      subarcs.

# P1

f. Sweeping and Sorting the spans:

Once all our profiles have  been computed, we begin the sweep to

build (and fill) the spans.

As  the TrueType specification  uses the  winding fill  rule, we

place  on each  scanline the  profiles present  in  two separate

lists.

One  list,  called  the  'left'  one,  only  contains  ascending

profiles, while  the other 'right' list  contains the descending

profiles.

As  each glyph  is made  of  closed curves,  a simple  geometric

property  is that  the two  lists necessarily  contain  the same

number of elements.

Creating spans is there straightforward:

1. We sort each list in increasing X order.

2. We  pair   each  value  of   the  left  list,   with  its

corresponding value in the right one.

/     /  |   |        For example, we have here four

/     /   |   |        profiles. Two of them are

>/     /    |   |  |     ascending (1 & 3), while the two

1//     /   ^ |   |  | 2   others are descending (2 & 4).

//     //  3| |   |  v

/     //4   | |   |          On the given scanline, the left

a /     /<       |   |          list is (1,3) and the right one

- - - *-----* - - - - *---* - - y -  is (4,2) (sorted).

/     / b       c|   |d

There are then two spans, joining

1 to 4 (i.e. a-b) and 3 to 2

(i.e. c-d)!

Sorting doesn't necessarily  take much time, as in  99 cases out

of 100, the lists' order is  kept from one scanline to the next.

We can  thus implement it  with two simple  singly-linked lists,

sorted by a classic bubble-sort, which takes a minimum amount of

time when the lists are already sorted.

A  previous  version  of  the  rasterizer  used  more  elaborate

structures, like arrays to  perform 'faster' sorting.  It turned

out that  this old scheme is  not faster than  the one described

above.

Once the spans  have been 'created', we can  simply draw them in

the target bitmap.

g. Drop-out control:

/* to be continued */