The G8BPQ Hostmode Emulator, and its use with NET

-------------------------------------------------

The current revision of NET (PE1CHL version) offers the option of emulating

a G8BPQ rev 4.xx NET/ROM switch.  This feature can be used to connect a BBS or

DX-Cluster to NET, offering an error-free connection of such a system to

the NET/ROM network and to local users.

The G8BPQ NET/ROM switch offers a simple hostmode interface, which allows

up to 8 applications to access the NET/ROM switch.  The popular F6FBB BBS

has the capability of using this hostmode interface.

Furthermore, G8BPQ has written a number of TSR programs that emulate existing

hostmode interfaces, like WA8DED, so that programs that do not directly

support the G8BPQ hostmode can be used too.

Now that NET can emulate the G8BPQ hostmode, it is possible to use these

same programs with the NET/ROM switch in NET.

The G8BPQ Hostmode Emulator for NET consists of two different parts:

1. A TSR program, G8BPQ.COM, offering the G8BPQ Hostmode interface to the

client applications (BBS, Cluster etc)

2. A part of NET.EXE which interfaces to G8BPQ.COM and handles the incoming

and outgoing connections.

The G8BPQ switch software package, version 4.06, is also required to setup

and understand the emulator.

Different configurations will be supported:

1. Multiple programs running under DESQview

----------------------------------------

Here, the Hostmode Emulator is loaded as a TSR *before* DESQview is loaded.

The emulator can therefore be called from each DESQview window.  NET is first

loaded in one of the windows, and the other applications can thereafter

be loaded in other windows.

The resulting hardware/software structure is:

***************************************************************

*                                                             *

*  +---------------+   +---------------+   +---------------+  *

*  |               |   |               |   |               |  *

*  |      NET      |   |   F6FBB BBS   |   |   DX-Cluster  |  *

*  |               |   |               |   |               |  *

*  +---------------+   +---------------+   +---------------+  *

*  +-------------------------------------------------------+  *

*  |                      DESQview/386                     |  *

*  +-------------------------------------------------------+  *

*  +-------------------------------------------------------+  *

*  |                 G8BPQ Hostmode Emulator               |  *

*  +-------------------------------------------------------+  *

*                                                             *

***************************************************************

386/486 system

2. Multiple PCs interconnected using a NetBIOS-compatible LAN

----------------------------------------------------------

In this mode, each application runs on its own PC, which does not need to

be a 386 system.  NET also runs on a pc of its own.

The PCs are interconnected using some LAN that provides a NetBIOS interface,

and the G8BPQ Hostmode Emulator is loaded on all PCs, after the NetBIOS

software has been loaded and initialized.

The resulting hardware/software structure is:

***********************  ***********************  ***********************

*                     *  *                     *  *                     *

*  +---------------+  *  *  +---------------+  *  *  +---------------+  *

*  |               |  *  *  |               |  *  *  |               |  *

*  |      NET      |  *  *  |   F6FBB BBS   |  *  *  |   DX-Cluster  |  *

*  |               |  *  *  |               |  *  *  |               |  *

*  +---------------+  *  *  +---------------+  *  *  +---------------+  *

*  +---------------+  *  *  +---------------+  *  *  +---------------+  *

*  |G8BPQ Hostmode |  *  *  |G8BPQ Hostmode |  *  *  |G8BPQ Hostmode |  *

*  |   Emulator    |  *  *  |   Emulator    |  *  *  |   Emulator    |  *

*  +---------------+  *  *  +---------------+  *  *  +---------------+  *

*  +---------------+  *  *  +---------------+  *  *  +---------------+  *

*  |  NetBIOS LAN  |  *  *  |  NetBIOS LAN  |  *  *  |  NetBIOS LAN  |  *

*  +---------------+  *  *  +---------------+  *  *  +---------------+  *

*                     *  *                     *  *                     *

***********************  ***********************  ***********************

|             	    |                         |

====================================================

LAN (Ethernet, ARCnet, Token-ring etc)

3. Mixed system

------------

It is also possible to mix these two methods, e.g. running BBS and Cluster

under DESQview on one system, and connect it using a LAN to another system,

which is running NET.

The TSR "G8BPQ.COM" should always be loaded *after* any NetBIOS software

has been loaded and initialized, and *before* NET, BBS or Cluster.

The currently released version of the G8BPQ Hostmode emulator does not

yet support the use of a LAN, so only the first configuration mentioned

above (Multiple programs running under DESQview) can be used right now.

How to define the access to the G8BPQ Emulator from NET

-------------------------------------------------------

NET will check for presence of one or more AX.25 ports in the range 11-18

to enable access to up to 8 different applications.  These port numbers

map to application numbers 1-8.  (Not very convenient, but port numbers 1-6

were already in use for other purposes)

G8BPQ has assigned the application numbers 1=BBS 2=SYSOP 3=REMOTE SYSOP,

but it really does not matter how you assign them....  You have to define

the ax25 ports only for the applications you want to run.  Example:

ax25 port 11 conn pi0bbs

ax25 port 12 conn pi0dxc

This will define 2 applications, #1 = pi0bbs (say, an F6FBB BBS), and

#2 = pi0dxc (say, a DX Cluster).  Of course you can select any callsign, but

the callsign/SSID combination should not be in use for other purposes in

the station.  It is also possible to restrict access to a certain interface,

by adding the name of that interface to the "ax25 port" command.

The access to the emulator is initialized using the "ax25 start g8bpq" command.

An optional argument to the "ax25 start g8bpq" statement lets you override

the default host interrupt number 127.  You should provide the interrupt number

when you have changed it in the BPQCFG file.  Example:

ax25 start g8bpq 126

NET will now check if the G8BPQ Hostmode Emulator is indeed loaded at the

specified interrupt number, and the versions of NET and Emulator are matching.

In addition, the NET configuration must include an active NET/ROM switch.

At least one of the "netrom interface" lines must define an interface as "lap",

so that it will be the default interface for outgoing connects.

Use of the G8BPQ Emulator

-------------------------

NET will accept incoming connects on the defined callsigns, and will pass them

through to applications that have set their application mask to include the

specified application number.  (this is explained in G8BPQ's docs)

Incoming connects can be either directly from an AX.25 station, or from a

station that was connected to the local NET/ROM and issued a "connect" command

with the proper callsign.

When an application asks the emulator for an outbound connection, it will be

connected to the NET/ROM command interpreter.  It will show up in the

userlist as "Host(# call)", where # is the stream number that the application

has passed to the emulator.

Now, the application can send any valid NET/ROM command, and it should issue

a "connect" command to connect to another station.

It is not possible for an application to directly connect an AX.25 station

without going through the NET/ROM.  This is because of the structure of the

G8BPQ hostmode.  Note that a program using the real G8BPQ software

encounters the same situation.

The sysop is responsible for assignment of the "application mask" and the

range of stream numbers being used by each application (see G8BPQ's docs).

When more than one application uses a given bit in the application mask, or

a stream number, strange things will happen.  This is not checked (and

cannot be checked) by the G8BPQ Hostmode Emulator.

Configuration file

------------------

Because G8BPQ documents the reading of the configuration file as the procedure

to find out the interrupt number to be used to access the switch, and some

of the G8BPQ-written programs use this file too, the "BPQCFG.BIN" configuration

file is also used by the G8BPQ Hostmode Emulator for NET.

The resident part of the emulator (G8BPQ.COM) reads the BPQCFG.BIN file when

it is loaded, and derives some of its parameters from the file's contents.

The file has to be in the current directory when G8BPQ.COM is loaded.

To construct a BPQCFG.BIN file, a textfile named BPQCFG.TXT is prepared using

a text editor, and it is converted using the utility BPQCFG.EXE written

by G8BPQ.   This utility is started without parameters, and converts

BPQCFG.TXT to BPQCFG.BIN in the current directory.

Only a few parameters are required by the emulator, but all have to be

specified for the BPQCFG.EXE program to work correctly.  In  the example

below, unused parameters have been moved to the end of the file, and have

been set to zero or blank values.

As the emulator also requires some information that the real G8BPQ switch

software does not need, some parameters are being used for a different

purpose than their name indicates.

The following parameters have special meaning to the emulator:

BBSALIAS	Used as NetBIOS name for a client emulator (on BBS system)

NODEALIAS	Used as NetBIOS name for a server emulator (on NET system)

BBSQUAL		Used as NetBIOS adapter number (usually will be 0)

MAXROUTES	Maximal number of client emulators that can connect to a server

PACLEN		Value of PACLEN reported to application when connected

L4WINDOW	Value of WINDOW reported to application when connected

MAXCIRCUITS	Number of streams supported

T3		Bitmask indicating which applications will have "transmit

packing", i.e. small packets sent by these applications will

be combined into larger packets up to PACLEN.

Example configuration file

--------------------------

;	For Version 4.06

;

;	CONFIGURATION FILE FOR G8BPQ SWITCH SOFTWARE

;

;	The program which converts this text file to binary

;	form is currently very crude - be VERY careful when

;	editing this file!

;

;	This configuration file has been stripped down to contain

;	only those parameters required for the G8BPQ emulator to

;	be used with PE1CHL-NET.  Processing it with BPQCFG.EXE

;	should result in a 3072-byte file, and no error messages.

;

;

;	Configuration Options

;

;	BBS and NODE flags are used like this:

;		System where BBS is running should have BBS=1  ('Client')

;		System where NET is running should have NODE=1 ('Server')

;

;	So, when running BBS and NET together under DesqView: BBS=1 and NODE=1

;	When using separate machines:	BBS machine has BBS=1 NODE=0

;					NET machine has BBS=0 NODE=1

;

BBS=1

NODE=1

;

;	The following four fields are only used in a LAN (NetBIOS)

;	environment. When using a single machine, leave the ALIAS fields blank.

;	ALIAS fields are used for NetBIOS names.  NODEALIAS always holds the

;	name of the NET machine.  BBSALIAS is only set at the machine(s)

;	which run the client applications (i.e. those that have BBS=1, NODE=0)

;	Each machine should have a unique name.

;	BBSQUAL is used to specify the "adapter number" of the NetBIOS card.

;	MAXROUTES is only used on the server, and specifies the maximum number

;	of client machines that can connect to the server.

;

BBSALIAS=

NODEALIAS=

BBSQUAL=0

MAXROUTES=0

;

;	NODECALL defines the default callsign, used on outbound connects

;	without explicit application (APPLMASK != 1)

;	BBSCALL defines the callsign used on outbound connects from

;	application #1

;

NODECALL=PE1CHL

BBSCALL=PE1CHL-11

;

HOSTINTERRUPT=127 ; Sets the Interrupt used to access BPQ Host Mode. Will

; normally be 127, but may be changed if this clashes with

; other software. BTRIEVE seems to use 127, so if you are

; using it, try INTERRUPT=126

;

DESQVIEW=1			; Set this to 1 when DesqView present, 0 if not

;

PACLEN=128			; REPORTED PACLEN

L4WINDOW=10			; REPORTED WINDOW

;

MAXCIRCUITS=64			; NUMBER OF STREAMS SUPPORTED

;

BUFFERS=255			; PACKET BUFFERS - 255 MEANS ALLOCATE AS MANY

; AS POSSIBLE - NORMALLY ABOUT 130, DEPENDING

; ON OTHER TABLE SIZES

;

T3=255				; This value determines for which applications

; transmit packing is present (i.e. multiple

; short packets will be combined)

;

;

;	'COMBIOS' Port definitions.

;

;	This section defines the virtual TNC's that the code supports.

;

;	The basic format is:

;

;	    TNCPORT

;		COM=n		; COMBIOS port number

;		APPLMASK=nn	; Defaults to 1 (BBS)

;	        APPLFLAGS=nn	; Defaults to 6

; 	    ENDPORT

;

;	See APPLS.DOC for details on use of APPLFLAGS and APPLMASK

;

;	Only TNC2 applications are supported through this interface.

;	For information on using KISS or DEDHOST applications, see file

;	DRIVERS.DOC

;

;  	Dont try to define more than 16 ports.

;

TNCPORT

COM=12

ENDPORT

;

TNCPORT

COM=13

APPLFLAGS=4		; NO 'CONNECTED' TO USERS

ENDPORT

;

;	Unnecessary parameters (for this purpose) that *have* to be

;	set to some value (or BPQCFG will complain).

;	We'll set them to dummy values...

;

APPLICATIONS=

BTINTERVAL=0

CTEXT:

***

EMS=0

ENABLE_LINKED=N

FULL_CTEXT=0

HIDENODES=0

IDINTERVAL=0

IDLETIME=0

IDMSG:

***

INFOMSG:

***

L3TIMETOLIVE=0

L4DELAY=0

L4RETRIES=0

L4TIMEOUT=0

MAXLINKS=0

MAXNODES=0

MINQUAL=0

NODESINTERVAL=0

OBSINIT=0

OBSMIN=0

PORT

ID=LOOPBACK

TYPE=INTERNAL

PROTOCOL=KISS

QUALITY=250

MAXFRAME=7

FRACK=7000

RESPTIME=1000

RETRIES=10

PACLEN=256

ENDPORT

ROUTES:

***

TRANSDELAY=0

UNPROTO=