Documentation -TFKISS- TheFirmware TNC Software Emulator

--------------------------------------------------------

by Mark Wahl, DL4YBG (last change: 20.10.96)

0. Introduction

---------------

0.1. Licenses

-------------

TFKISS is based on TheFirmware by NORD><LINK, license policy given by

'copyrght.txt' in this archive. ALAS is a german license for ham radio

software.

Most everything of the AXIP-interface is taken from AX25IP written by

Mike Westerhof KA9WSB. His copyright notice is given below:

>-------------------------------------------------------------------------

>Copyright 1991, Michael Westerhof, Sun Microsystems, Inc.

>This software may be freely used, distributed, or modified, providing

>this footer is not removed.

>-------------------------------------------------------------------------

The PID change code was written together with Joachim, DL8HBS.

The FLEXNET decoding code was written by Rene, DG0FT.

The other code and all adaptions are written by me. I put this code

under license policy of ALAS, too.

0.2. Description

----------------

TNT, my hostmode terminal program needs a TNC with TheFirmware connected

on a serial interface. There are some applications, where other

configurations are needed:

- The TNC or PR-controller software is not compatible to TheFirmware and

its hostmode, but does support KISS mode

- The interface is not on a HF-frequency, but a TCP/IP-based network

(local net or internet)

- TNT together with DPBox shall be used directly connected to a digipeater

using KISS mode (hopefully with CRC) as interface

To handle these applications, TFKISS was written. It is in fact a porting

of the original TheFirmware by NORD><LINK, written for TNC2-hardware.

The low level interfaces were changed and adapted to Linux. On the

radio side a KISS- and an AXIP-interface take the place of the HDLC/modem

interface. The host connection is changed to a direct console interface (for

terminal mode) and a socket interface (for TNT).

1. Configuration

----------------

It is possible to operate TFKISS in several combinations. The radio side

can be a KISS interface, an AXIP interface or a combination of both.

On the host side a simple terminal mode can be used (nice for first tests)

or a socket interface to work together with TNT.

1.1. General configuration (tfkiss.ini)

---------------------------------------

This file is searched in the current directory and if not found in the

home directory of the user.

Here is a description of parameters which can be defined in the main

configuration file.

a) general parameters

- kiss_active		must be set to 1, if KISS interface shall be used,

otherwise set to 0.

- axip_active		must be set to 1, if AXIP interface shall be used,

otherwise set to 0.

- tfkiss_dir		directory path where all tfkiss files, configuration

and data/log files will be stored.

- tfkiss_procfile	name of file containing the process id of the running

TFKISS process (for use with BOXSTART).

- tfkiss_errfile	name for logfile to which logging data, warning and

error messages will be saved.

- tfkiss_parafile	name of file where TFKISS will store internal para-

meters (callsign, txdelay, e.t.c.) at termination.

Only one of the following values is allowed:

- tfkiss_socket		defines name of socket which is used as an interface

to TNT. The socket is defined by a valid pathname.

If the name does not start with a '/', 'tfkiss_dir'

is added at the beginning.

- tfkiss_extsocket	defines name of socket which is used as an interface

to TNT. The socket can be an UNIX- or an IP-Socket.

Unix-sockets are defined by 'unix:<pathname>',

IP-Sockets by <IP-address/name>:<portnumber>.

Valid examples are:

unix:tfkiss

unix:/usr/local/dptnt/tfkiss/tfkiss

*:5000

localhost:5002

bla.blub.org:6000

If both tfkiss_socket and tfkiss_extsocket are left blank, TFKISS will start

in terminal mode on the console.

b) parameters for KISS interface

- device		device used for a KISS interface, a serial device or

any other tty-device.

- tfkiss_lockfile	lockfile to lock the KISS device against other

programs.

- speed			baudrate used on KISS device

- fulldup_on_dama	set to 1 for better DAMA performance, allows a quick

answer on the poll of the DAMA master independent of

DCD.

- kisstype		type of KISS which shall be used on KISS device:

0: normal KISS without CRC

WARNING: This is HIGHLY insecure! You may lose

characters on the serial line WITHOUT knowing it.

Use this only for testing purposes!

1: SMACK, KISS with CRC16, developed by

Jan Schiefer, DL5UE

2: RMNC-KISS, KISS with CRC as implemented in

FLEXNET digipeater software

Note: There is NO autoprobing of the protocol

of the connected KISS device. You have to select

the correct mode manually!

c) parameters for AXIP interface

- tfkiss_axipconfig	name of file which will contain all parameters

concerning AXIP configuration.

Sample tfkiss.ini file:

# enable (1) or disable (0) kiss port

kiss_active 1

# enable (1) or disable (0) axip port

axip_active 0

# device for kiss port

device /dev/cua0

# lockfile for device used for kiss port

tfkiss_lockfile /usr/spool/uucp/LCK..cua0

# speed on device for kiss port

speed 19200

# switch kiss-tnc to fullduplex if DAMA detected

fulldup_on_dama 1

# directory for config/status files

tfkiss_dir /work/tfkiss

# file containing process id

tfkiss_procfile tfkiss.pid

# log file for errors

tfkiss_errfile tfkiss.err

# save file for parameters

tfkiss_parafile tfkiss.par

# UNIX-socket for connection to TNT

tfkiss_socket socket

# config-file for axip port

tfkiss_axipconfig tfkiss.cfg

# type of KISS on kiss port: 0: KISS, 1: SMACK, 2: RMNC-KISS

kisstype 1

<EOF>

1.2. AXIP configuration (tfkiss.cfg)

------------------------------------

All AXIP configuration is defined in this file, which layout is similar

to that of the original AX25IP program:

- socket		give mode of operation, several entries are

possible.

ip: AXIP according to RFC1226

udp: transport of AX25 data via an udp connection,

if no port is given, 10093 is used as a

default.

- mode			defines the mode of operation of the AXIP part

of TFKISS.

tnc: data is given transparently to/from network

digi: data is digipeated using call defined as

mycall

- mycall		digipeater callsign, only needed if mode digi is

used.

- loglevel		logging function for debugging purposes, logging

data will be put to file defined by tfkiss_errfile.

- route			defines routing data, gives network destination

and transport type for AX25 callsigns.

syntax: route <callsign> <hostname> [udp] [udp-port]

For default route <callsign> is substituted by

'default'.

Note: If default route is defined, no data will use

an available KISS interface!

sample tfkiss.cfg file:

# Sample tfkiss/ax25ip configuration file

#

# First select active socket types

# syntax: socket <ip/udp> [udp-port]

#

socket ip

socket udp

#

# Select the mode of operation. (digi or tnc)

#

mode tnc

#

# If you selected digi, you must define a callsign.  If you selected

# tnc mode, the callsign is optional

#

mycall nocall

#

# loglevel 0 - no output

# loglevel 1 - config info only

# loglevel 2 - major events and errors

# loglevel 3 - major events, errors, and AX25 frame trace

# loglevel 4 - all events

#

loglevel 0

#

# Define some routes.  One example routes all traffic for callsign db0zzz

# to a host named nexthost.bla.blub.  You can define as many as

# required.

# syntax: route <callsign> <hostname> [udp] [udp-port]

#

route db0zzz nexthost.bla.blub

route db0zzy-5 otherhost.bla.blub udp

route db0zzx-2 localhost udp 10094

#

# A catch-all is provided: this line sends all calls not specifically

# noted in the routing tables to otherhost.bla.blub.  Use this feature

# with great care -- the host on the other end may not appreciate all the

# traffic!

# syntax: route default <hostname> [udp] [udp-port]

#

#route default otherhost.bla.blub

#

<EOF>

2. Options at startup

---------------------

Some options can be given at startup of tfkiss to change the behaviour

of the program.

Usage : tfkiss [-i <init-file>] [-s <tfkiss-socket>] [-u] [-r] [-t]

a) -i	defines a different init-file instead of tfkiss.ini

b) -s	defines a socket for the interface with TNT, even if tfkiss_socket

is not defined

c) -u	ignores an existing lockfile for the KISS device

d) -r	ignore and delete all parameter values stored in tfkiss_parafile

e) -t	startup on console instead of using the socket to TNT, even

if tfkiss_socket is defined

3. Finish TFKISS

----------------

To finish TFKISS two ways are possible:

In terminal mode give the command QRES, which, in contrast to the original

TNC-software, will only terminate the program, all parameter values like

callsign, txdelay, e.t.c. will be saved in tfkiss_parafile for the next

run.

If the socket interface to TNT is used, simply send a SIGTERM signal to

the process ('kill <tfkiss-process-ID>').

4. Extensions to TheFirmware

----------------------------

4.1. Parameter for txtail-length

--------------------------------

In KISS mode a value for the time between the last character put in the

HDLC controller and the switch off of the transmitter can be given.

This time is called 'txtail'.

It can be handled by the new extended TNC command '@TA'. The metric of

the value is 10ms, 4 means a txtail of 40ms.

4.2. Parameter for changing PID

-------------------------------

For special applications, like broadcast or TCP/IP different PID values

are needed. The PID can be changed using the new extended TNC command '@P'.

The argument for this command is the PID value in decimal. The PID will

be changed only on the currently selected channel.

4.3. Information about a changed PID

------------------------------------

If the remote station use a different PID than the default one 240 (0f0h),

a link status message will inform about this:

'PID changed to (EF)'

4.4. Decoding of frames in internal FLEXNET protocol

----------------------------------------------------

FLEXNET digipeaters use a special format for communication. Only one address

field (7 Bytes) is used to define source and destination of the data.

This specially encoded address field is displayed in readable form.

5. Contact

----------

If you want to get in contact with me use the following ways:

HAM RADIO: DL4YBG @ DB0GR.#BLN.DEU.EU

E-Mail:    wahlm@berlin.snafu.de

Internet:  http://www.snafu.de/~wahlm/

73, Mark, DL4YBG